# Number system - Hexa-decimal number (Base-16)

Any numerical value (number) can be represented in any of the following number system.

| Number Base | Number system |  |
| --- | --- | --- |
| Base 2 | binary | The common representation of data in computer is a string of bits, using the binary numeral system (Base-2). |
| Base 8 | octal |  |
| Base 10 | decimal | The most familiar representation of numbers is in base 10 systems. Eg 12345. |
| Base 16 | hexadecimal | Binary strings (01000001) are long and hard for human programmers to read. To simplify this, programmers split a byte in half (two 4-bit sections called nibbles) and convert them into Hexadecimal (Base-16). It is convenient shorthand notation to talk about binary numbers. Hexadecimal is also commonly used to represent computer memory addresses. |

Hexadecimal is a number system that uses 16 symbols to represent numbers.

- Hexadecimal uses the symbols 0–9 to represent the values 0–9.

- It uses the letters A–F to represent the values 10–15. In hexadecimal, when you reach the decimal number 10, you switch to letters A through F because a single digit can hold any value up to 15.

- Each digit represents a decimal value. For example, D is equal to 13 in base 10.

## Conversion between number systems

| Decimal to Hexa-decimal conversion | Hex to decimal conversion |
| --- | --- |
| Identify the group of 4 bits each. Fill the partial group with 0’s on outside. Then replace each group with the corresponding hex digit. | hexadecimal number (Base-16) 0x2AF3 is equal to 10,995 decimals (base-10). |
| 0011; 1010; 0101; 0110; .; 1110; 0010; 1111; 1000. 3; A; 5; 6; .; E; 2; F; 8. | Base-16; 2; A; F; 3. base-16 to base-10; 2×163; A×162; F×161; 3×160. 2×163; 10×162; 15×161; 3×160. 2x4096 =8192; 10x256 =2560; 15x16 =240; 3*1 =3. Base-10; 10,995. |

| 8-Bits (1byte) | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Base -2 place holder | 1*27 = (128) | 1*26 = (64) | 1*25 = (32) | 1*24 = (16) | 1*23 = (8) | 1*22 = (4) | 1*21 = (2) | 1*20 = (1) |
| Decimal value | 255 |
| Hexadecimal value | 0xFF |